JLF.00.001 – GLUE THREAD JOINTING MACHINE WITH  EXTRACTOR,

JLF MODEL

Description

Glue Thread Jointing Machine, zig-zag seam and coupling system through flat discs drawn by worm type reduction gear/oil-bath lubricated crown gear driven by a ¾HP electric motor (working speed = 28m/min)

- Extracting assembly: smooth extracting roller drawn by ⅓ HP reduction gear motor assembly and pressing roller with pressure adjustment.

- Direct current quick-stop drive assembly brake built in motor

- Two-plane table with aluminum surface and central arm, allowing gluing of laminated wood of unlimited width

- Glue thread pressing through liquid Vaseline coated metal roller to avoid gluing of thread on the surface

- Short-circuit and overload protected electric system.

- Rocking link glue thread laying nozzle governed by alternate current motor

Glue thread melting system: governed by 10/12/14/16 voltage switch for electric resistor of 300 watts and 40/35/30/25 amperes.