JLP.00.001 – GLUE TAPE JOINTING MACHINE WITH EXTRACTOR,

JLP MODEL

SPECIFICATIONS

Glue Tape Jointing Machine and coupling system by means of flat discs drawn by worm type reduction gear and oil-bath lubricated bronze crown gear, driven by a ¾HP electric motor (working speed 30m/min).

-        Direct current quick-stop drive assembly brake built in motor

-        Two-plane table with aluminum surface and central arm, allowing gluing of laminated wood of unlimited width

-        Glue tape pressing through metal roller and electromagnetic brake system for tape cut-off.

-        Short-circuit and overload protected electric system.

-        Tape feeder.

-        Reel holder assembly with quick coupling reel change system and glue cure timing

-        Automatic tape cut-off through extracting roller system, driven by a ⅓HP reduction gear motor, pressing roller with metal surface and pressure adjustment.